Name:												   Date:

Competency Statement: The perioperative RN has completed facility or health care organization–required education and competency verification activities related to minimally invasive surgery (MIS).1

- Guideline for minimally invasive surgery. In: Guidelines for Perioperative Practice. Denver, CO: AORN, Inc.

Outcome Statement: The patient is free from injury related to perioperative equipment, medical supplies, or instrumentation.

| Competency Statements/Performance Criteria |  | Verification Method [Select applicable code from legend at bottom of page] |  |  |  |  |  | Not Met (Explain why) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DEM/ DO/DA | KAT | S/SBT/ CS | V | RWM/ P&P | O |  |
| Safe Environment |  |  |  |  |  |  |  |  |
| - Reviews the manufacturers’ instructions for use (IFU) for all MIS equipment and instruments used during procedures. |  |  |  |  |  |  |  |  |
| - Prepares MIS equipment and surgical instruments according to the manufacturer’s IFU and surgeon preference. |  |  |  |  |  |  |  |  |
| - Verifies the functionality of equipment that will be used during the procedure according to the manufacturer’s IFU. |  |  |  |  |  |  |  |  |
| - Inspects MIS instruments before use. |  |  |  |  |  |  |  |  |
| - Removes defective items from service, and sends items for repair or replacement according to [facility-specific policy]. |  |  |  |  |  |  |  |  |
| - Identifies and verifies the availability of resources needed in the event of conversion to an open procedure. |  |  |  |  |  |  |  |  |
| Gas Distension Media |  |  |  |  |  |  |  |  |
| - Mitigates the risk for injuries related to gas distension media by - elevating the insufflator above the level of the surgical cavity, when possible; |  |  |  |  |  |  |  |  |
| - setting the insufflation flow rate according to the manufacturer’s IFU and surgeon’s specification; |  |  |  |  |  |  |  |  |
| - setting, monitoring, and maintaining the gas insufflation pressure at the lowest level necessary to achieve pneumoperitoneum within the specification of the surgeon; |  |  |  |  |  |  |  |  |
| - flushing the insufflator and tubing with the selected gas before connecting the tubing to the patient cannula (eg, Veress needle); |  |  |  |  |  |  |  |  |
| - verifying the presence of a hydrophobic filter between the insufflator and insufflation tubing and that the filter is compatible with the insufflator used; |  |  |  |  |  |  |  |  |
| - turning on gas insufflator alarms and making them sufficiently audible above ambient OR noise; and |  |  |  |  |  |  |  |  |
| - verifying before the procedure begins that gas cylinders - contain the selected gas, |  |  |  |  |  |  |  |  |
| - are not empty, and |  |  |  |  |  |  |  |  |
| - have full back up cylinder(s) readily available for immediate replacement. |  |  |  |  |  |  |  |  |
| Fluid Distension Media |  |  |  |  |  |  |  |  |
| - Prepares irrigation or fluid distension media according to the surgeon’s order and based on procedure type, patient assessment, and instruments used (eg, energy devices). |  |  |  |  |  |  |  |  |
| - Mitigates the risk for injury related to fluid used for irrigation or distension media by - monitoring the amount of fluid dispensed and collected during hysteroscopies and other procedures according to [facility-specific policy]; |  |  |  |  |  |  |  |  |
| - when fluid deficit monitoring is indicated for a procedure, determining in collaboration with the team regular intervals for reporting the fluid deficit throughout the procedure and reporting at these intervals; |  |  |  |  |  |  |  |  |
| - determining in collaboration with the team, patient-specific maximum hysteroscopic fluid deficit thresholds and what interventions should be taken when specified maximum thresholds are reached (eg, continued monitoring, termination of procedure); |  |  |  |  |  |  |  |  |
| - collecting as much fluid as possible in a closed-container system; |  |  |  |  |  |  |  |  |
| - preventing fluid from coming into contact with electrical equipment; |  |  |  |  |  |  |  |  |
| - using automated fluid management systems according to the manufacturer’s IFU; |  |  |  |  |  |  |  |  |
| - verifying that automated fluid management system alarms are on and audible above ambient OR noise; and |  |  |  |  |  |  |  |  |
| - verifying the fluid pump settings for fluid distension with the surgeon and monitoring the settings throughout the procedure. |  |  |  |  |  |  |  |  |
| Computer-Assisted Navigation and Robotics |  |  |  |  |  |  |  |  |
| - Safely positions the patient to facilitate use of computer-assisted navigation equipment or robotic-assisted equipment. |  |  |  |  |  |  |  |  |
| - Delegates, supervises, and evaluates the activities for setting up computer-assisted navigation equipment and robotic-assisted equipment, including - assisting with the navigation system registration process by |  |  |  |  |  |  |  |  |
| - verifying availability of preoperative imaging studies; |  |  |  |  |  |  |  |  |
| - positioning the navigation equipment so the surgical field is in view of the tracking system; |  |  |  |  |  |  |  |  |
| - attaching the patient tracker, if needed; |  |  |  |  |  |  |  |  |
| - collecting data points from anatomical landmarks or fiducial markers; |  |  |  |  |  |  |  |  |
| - registering additional instruments, if needed; and |  |  |  |  |  |  |  |  |
| - verifying the accuracy of the registration; and |  |  |  |  |  |  |  |  |
| - safely docking and undocking the robot during robotic-assisted procedures. |  |  |  |  |  |  |  |  |
| - Notifies the surgeon immediately if the navigational tracking devices are dislodged after registration during computer-assisted navigation procedures or if the patient moves after docking during robotic-assisted procedures. |  |  |  |  |  |  |  |  |
| Patient Care, Assessment, and Monitoring |  |  |  |  |  |  |  |
| - Preoperatively assesses the patient to identify risk for complications related to MIS, including fluid distension media. |  |  |  |  |  |  |  |
| - Monitors the patient for signs of a gas embolism. |  |  |  |  |  |  |  |
| - Describes possible interventions for gas embolism. |  |  |  |  |  |  |  |
| - Monitors the patient for physiologic changes in core temperature; laboratory test values; vital signs; and fluid retention in the abdomen, face, and neck. |  |  |  |  |  |  |  |
| - Assesses the patient for signs and symptoms of injury and complications related to the use of computer-assisted navigation and robotic-assisted equipment. |  |  |  |  |  |  |  |
| - Documents care provided during MIS procedures according to [facility-specific policy]. |  |  |  |  |  |  |  |
| - Locates, reviews, and asks clarifying questions about the policy and procedure for MIS. |  |  |  |  |  |  |  |
| - Advocates for patient safety by speaking up when there is a discrepancy or concern. |  |  |  |  |  |  |  |
| - Reports near misses and adverse events per [facility-specific policy]. |  |  |  |  |  |  |  |
| - Supports education and quality improvement initiatives related to MIS. |  |  |  |  |  |  |  |

| Concurrent competency verification of the following is recommended |  |
| --- | --- |
| - [Additional competencies related to MIS and computer-assisted procedures as determined by the facility or health care organization.] |  |